- How to check if MDVR can not find RS485 of Passenger Counter

- There are some reasons if the MDVR can not find RS485, such as the following possible reasons.

- 485 cables connection wrong: MDVR 485-A should connect 485-A of Passenger Counter, 485-B should connect 485-B of Passenger.

- 485 serial port option selection is wrong:Need select JHY CNT VER2

- 24pin(Alarm Cable) is not working:Can change the alarm cable and test

- Firmware version can not issue:Firmware version not support Passenger Counter, need upgrade to the special version, can confirm with the Support Engineer.

- 1.For the Passenger Counter host, if the host address code is 0 as default, the MDVR will not find the RS485 serial port. It shows like the following picture. It shows “None”, but if it works well, it will shows “Exist”.

- 2.First you can connect to the Passenger Counter host to the PC using RJ45 port of the counter.Use switch and RJ45 cable.

- Check the parameter settings, compare the Device Address, if it’s 0.

- 4.If it’s 0, need change the value to 1, and if you need connect two Passenger Counters, the front door and back door, you can set 1 for front Counter host, and set 2 for back Counter host.