- axis sensor introduction

- 1.Introduction

- MPU-60X0 is the world's first integrated 6-axis MotionTracking device. It integrates a 3-axis MEMS gyroscope, a 3-axis MEMS accelerometer, and an extensible digital motion processor DMP (Digital Motion Processor), which can be connected to a third-party digital sensor, such as a magnetometer, with an I2C interface. After expansion, it can output a 9-axis signal through its I2C or SPI interface (SPI interface is only available on MPU-6000). MPU-60X0 can also connect non-inertial digital sensors, such as pressure sensors, through its I2C interface.

- The MPU-60X0 uses three 16-bit ADCs for the gyroscope and accelerometer, respectively, and converts the analog quantity it measures into a digital output. In order to accurately track fast and slow movements, the measurement range of the sensor is user-controllable, the gyro measurement range is ±250, ±500, ±1000, ±2000°/sec (dps), and the accelerometer measurement range It is ±2, ±4, ±8, ±16g.

- An on-chip 1024-byte FIFO helps reduce system power consumption. Communication with all device registers uses 400kHz I2C interface or 1MHz SPI interface (SPI is only available for MPU-6000). For applications that require high-speed transmission, a 20MHz SPI can be used to read and interrupt registers. In addition, a temperature sensor and an oscillator with only ±1% variation in the working environment are embedded on the chip. The chip size is 4×4×0.9mm, using QFN package (leadless square package), which can withstand a maximum impact of 10000g, and has a programmable low-pass filter.

- Regarding power supply, MPU-60X0 can support VDD range 2.5V±5%, 3.0V±5%, or 3.3V±5%. In addition, the MPU-6050 also has a VLOGIC pin, which is used to provide logic levels for the I2C output. The VLOGIC voltage can be 1.8±5% or VDD.

- Features

- Digital output of 6-axis or 9-axis rotation matrix, quaternion (quaternion), Euler angle format (EulerAngleforma) fusion calculation data.

- 3-axis angular velocity sensor (gyroscope) with 131 LSBs/°/sec sensitivity and full-scale sensing ranges of ±250, ±500, ±1000, and ±2000°/sec.

- Programmable control, and the program control range is ±2g, ±4g, ±8g and ±16g 3-axis accelerator.

- Remove the sensitivity between the accelerator and the gyroscope axis to reduce the effect of the setting and the drift of the sensor.

- The Digital Motion Processing (DMP: Digital Motion Processing) engine can reduce the load of complex fusion calculation data, sensor synchronization, and posture sensing. The motion processing database supports the built-in operating time deviation and magnetic sensor calibration calculation technology of Android, Linux and Windows, eliminating the need for customers to perform additional calibration.

- Digital output temperature sensor

- Digital input sync pin (Syncpin) supports video electronic video stabilization technology and GPS

- Programmable interruption supports gesture recognition, panning, zooming in and out of the screen, scrolling, rapid descent interruption, high-G interruption, zero motion sensing, touch sensing, and shaking sensing functions.

- VDD power supply voltage is 2.5V±5%, 3.0V±5%, 3.3V±5%; VDDIO is 1.8V±5%

- Gyroscope operating current: 5mA, gyroscope standby current: 5uA; accelerator operating current: 500uA, accelerator power saving mode current: 40uA@10Hz

- I2C in fast mode up to 400kHz, or SPI serial host interface up to 20MHz

- The built-in oscillator has a frequency variation of only ±1% within the operating temperature range. Optional external clock input 32.768kHz or 19.2MHz

- Function introduction

- Pin output and signal description

- Typical usage